# USER  GUIDE

- Firstly you need install pcie driver and initialize DDR(load ZSP firmware and run it).

- Compile source code: make

- Test pcie bandwidth: #./pcie_bw_test   dev  rc2ep / ep2rc   test_count   size

dev:  device node;  for example: /dev/transcoder0

test_count :  edma transmission times.

size: The size of each transmission, unit is MB, maximum is 64.

Edma will transmit [test_count] times, every time is [size] MB, and then output the bandwidth average.

For example:

Transmit data from host to EP, every time size is 2MB, round is 10.

# ./pcie_bw_test   /dev/transcoder0   rc2ep   10   2

device node is /dev/transcoder0

RC address=0x10500000

EP address=0x4200000

normal edma trans size=0x200000, count=10..

round 0 time=2615us.

round 1 time=2612us.

round 2 time=2610us.

round 3 time=2611us.

round 4 time=2611us.

round 5 time=2611us.

round 6 time=2607us.

round 7 time=2607us.

round 8 time=2608us.

round 9 time=2607us.

total_time=26099 us, avg_bw= 766.000000 MB/s

- Test DDR stability: # usage: ./pcie_ddr_memtest <d:device> [r | w | t] [i] [E] [n] [s] [S] [I] [filename]

-d: device node; e.g. /dev/transcoder0

-r: read data from ep ddr

-w: write data to ep ddr

-t: do trans test, default test

-s: size for each edma trans, default 8m

-E: ep end address, default s0:0x70000000 s1:0xF000000

-S: select slice, default 0

-i: input file.

-I: use interrupt, default polling

-n: loops

#./pcie_ddr_memtest  –d device_node -I	//default test s0, use 8M size, no loops

#./pcie_ddr_memtest  –d device_node	 -S1 -I	//test s1, use 8M size, no loops

example:

#./pcie_ddr_memtest -d /dev/transcoder0 -I

[memtest] testing device node is /dev/transcoder0.

[memtest] trans 0 begin, ep addr: 0x04000000, size: 00800000

[memtest] compare pass start addr: 0x04000000, end addr: 0x80000000 !!!

[memtest] dump data ...